SIMATS ENGINEERING

ASSESSMENT TOOL 3

COURSE NAME: Computer Vision with OpenCV        COURSE CODE: DSA0202

COURSE OUTCOME COVERED

CO2: Apply image processing and feature extraction techniques, including edge  detection, motion estimation, and optical flow, for analyzing images.. (BL3)

Assessment Tool Weightage: 20%

QUESTIONS: 2 Total Marks:20 Annexure A

Class Test

Code of Conduct:

I N.JOE JERISH(192524374) certify that this submission is my original work and  that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action. Signature of the student:

RUBRICS FOR CALCULATION

| Criteria | Weighta ge | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning | Marks |
| --- | --- | --- | --- | --- | --- | --- |
| Understand ing of Concepts | 25% | Demonstrates thorough, accurate understanding of concepts | Good understanding with minor gaps | Basic understanding with some misconceptions | Poor understanding; major misconceptions |  |
| Explanatio n | 25% | Detailed, well explained answers with relevant examples | Clear explanation with adequate details | Limited explanation; lacks depth | Poor or unclear explanation |  |
| Applicatio n | 20% | Effectively applies concepts with strong analytical ability | Applies concepts with minor errors | Limited application with weak analysis | No application or incorrect analysis |  |
| Organizati on | 15% | Well-structured answers with logical flow and coherence | Organized with minor issues in flow | Basic structure, lacks clarity | Poor organization, incoherent answers |  |
| Accuracy | 10% | Completely accurate and covers all required points | Mostly accurate with minor omissions | Partially correct with missing points | Incorrect answers with major gaps |  |
| Clarity | 5% | Clear, neat, professional presentation | Understandable with minor issues | Limited clarity, readability issues | Poorly written, difficult to understand |  |

Annexure A

Class Test

Question 1: Image Preprocessing and Feature Extraction

Scenario: A drone is used to capture aerial images of a farmland to identify pest affected crop regions. Due to sensor limitations, wind movement, and changing  sunlight, the captured images contain noise, shadows, and uneven illumination. Pest affected areas usually show visible texture and color variations compared to healthy  crop regions.

Answer the following questions:

a) Preprocessing Pipeline — 4 Marks

Propose a suitable preprocessing pipeline to improve the quality of the farmland  images before analysis. Your answer should include methods for noise reduction,  illumination correction, contrast enhancement, and normalization. Justify the purpose  of each step.

b)Feature Extraction — 3 Marks

Choose an appropriate feature extraction method to identify texture changes caused  by pest damage. Explain why the selected method is suitable for detecting affected  crop regions.

c)Rotation and Scale Variations — 3 Marks

Drone images may be captured from different heights and angles. Discuss how rotation  and scale variations can affect feature detection. Suggest suitable methods to make  the feature extraction process more robust.

Question 2: Stereo Vision and Motion Estimation

Scenario: A self-driving car is equipped with stereo cameras to navigate through a busy  street. The system must estimate the distance of nearby vehicles and pedestrians in  real time. It should also track moving objects accurately to avoid collisions, especially  when pedestrians or vehicles change speed suddenly.

Answer the following questions:

a) Stereo Disparity and Depth Estimation — 4 Marks

Explain how stereo cameras use the difference between left and right image views to  estimate the depth of objects. Discuss how disparity changes for nearby and distant  objects.

b) Optical Flow for Motion Tracking — 3 Marks

If a pedestrian suddenly changes speed or direction, explain how optical flow helps in  tracking the pedestrian’s motion between consecutive video frames. c) Errors and Mitigation Techniques — 3 Marks

Identify possible sources of error in depth estimation and motion tracking, such as  camera misalignment, poor lighting, occlusion, fast motion, or textureless regions.  Suggest suitable techniques to reduce these errors.

# Question 1 — Image Preprocessing and Feature Extraction

Scenario: A drone captures aerial farmland images to identify pest-affected crop regions. The images contain noise, shadows, and uneven illumination caused by sensor limitations, wind movement, and changing sunlight. Pest-affected areas show visible texture and color variation compared to healthy crop regions.

a) Preprocessing Pipeline

A four-stage pipeline addresses the noise, shadow, and illumination problems in sequence, since each stage assumes a relatively clean input from the one before it:

- 1. Noise Reduction — A bilateral filter (or median filter for salt-and-pepper sensor noise) is applied first. Unlike a simple Gaussian blur, the bilateral filter smooths flat regions while preserving crop-edge boundaries, which is essential because edge and texture information is exactly what later pest-detection steps depend on.

- 2. Illumination Correction — Shadows and uneven sunlight are corrected using CLAHE (Contrast Limited Adaptive Histogram Equalization) applied on the luminance (L) channel of the image in LAB color space, or by homomorphic filtering. This equalizes brightness across the field so that a healthy leaf in shadow is not misclassified as diseased simply because it appears dark.

- 3. Contrast Enhancement — Global or adaptive histogram equalization boosts the contrast between healthy green tissue and pest-affected patches (which are typically yellow/brown or textured differently), making subtle color and texture differences easier for the next stage to detect.

- 4. Normalization — Pixel intensities are rescaled to a common range (e.g., 0–1) and color values are normalized (e.g., white-balance / z-score normalization per channel). This ensures that images captured at different times of day or under different lighting conditions are numerically comparable, which is important since the same pest region should produce similar feature values regardless of when it was photographed.

Figure 1: Proposed preprocessing pipeline for the farmland images.

b) Feature Extraction

Texture-based feature extraction is best suited here because pest damage manifests as a change in surface texture (spots, lesions, rough patches) rather than just a simple color shift.

- Recommended method: Gray-Level Co-occurrence Matrix (GLCM). GLCM computes second-order statistics — contrast, homogeneity, energy, and correlation — from the spatial relationship between neighboring pixel intensities.

- Why it suits this scenario: Healthy crop regions have a fairly uniform, repetitive texture, while pest-affected regions show irregular intensity variation. GLCM contrast and homogeneity values change measurably between these two cases, making it possible to numerically separate affected from healthy regions.

- Alternative: Local Binary Patterns (LBP) can also be used — it is computationally lighter and is robust to the illumination changes typical of outdoor drone imagery, making it a practical complement to GLCM.

c) Rotation and Scale Variations

Because the drone captures images from different heights and angles, the same crop region can appear at different sizes (scale) and orientations (rotation) across images.

- Effect on detection: Basic corner detectors (e.g., Harris corner detector) and raw texture descriptors are not scale- or rotation-invariant. A pest patch that is correctly detected at one altitude/angle may go undetected at another because its apparent size or orientation has changed, reducing detection consistency across the dataset.

- Recommended fix: Use scale- and rotation-invariant feature detectors/descriptors such as SIFT (Scale-Invariant Feature Transform) or ORB (Oriented FAST and Rotated BRIEF). These build descriptors that are normalized for local scale and orientation, so the same crop feature is matched reliably regardless of drone height or heading.

- Additional measure: Constructing an image pyramid (multi-scale representation) before feature extraction, combined with geo-referencing/altitude metadata from the drone, further improves robustness by allowing features to be compared at a consistent effective ground scale.

# Question 2 — Stereo Vision and Motion Estimation

Scenario: A self-driving car with stereo cameras must estimate the distance to nearby vehicles and pedestrians in real time, and must track moving objects accurately, especially when they change speed suddenly.

a) Stereo Disparity and Depth Estimation

Stereo cameras are mounted a fixed distance apart (the baseline, b). Because of this offset, the same real-world point projects to slightly different horizontal pixel positions in the left and right images — this horizontal pixel-position difference is called disparity (d).

- Depth is recovered using the relation Z = (f × b) / d, where f is the focal length of the camera, b is the baseline distance between the two cameras, and d is the disparity (in pixels) for that point.

- Nearby objects produce a large disparity — their projected position shifts significantly between the left and right images — while distant objects produce a small disparity, since parallax decreases with distance.

- Because disparity is inversely proportional to depth, small errors in disparity matter far more for distant objects than for nearby ones, which is why accurate calibration and precise pixel-level correspondence matching (stereo matching) are critical for reliable depth estimation at range.

Figure 2: Disparity decreases as object distance from the stereo camera pair increases.

b) Optical Flow for Motion Tracking

Optical flow estimates the apparent motion of pixel intensities between two consecutive video frames, based on the brightness-constancy assumption (a pixel's intensity stays the same as it moves, only its position changes).

- When a pedestrian suddenly changes speed or direction, the optical flow vectors (u, v) computed at the pedestrian's location change in magnitude and direction accordingly, allowing the system to detect the change in motion almost immediately, frame to frame.

- Sparse methods such as Lucas–Kanade optical flow track a set of distinctive keypoints (e.g., corners on the pedestrian's silhouette) efficiently in real time, while dense methods such as Farneback optical flow compute a flow vector for every pixel, giving a fuller picture of the motion field at higher computational cost.

- This continuous vector field lets the self-driving car predict the pedestrian's next position and react (brake/steer) before a collision risk becomes critical.

Figure 3: Optical flow vector describing pixel displacement between consecutive frames.

c) Errors and Mitigation Techniques

Several practical factors can degrade depth estimation and motion tracking accuracy:

- Camera misalignment: Even slight physical misalignment of the stereo pair introduces incorrect disparity values. Mitigation: regular stereo calibration and image rectification so that corresponding points lie on the same horizontal scanline.

- Poor lighting: Low light or glare reduces image contrast, making correspondence matching and flow estimation unreliable. Mitigation: illumination normalization/preprocessing, and sensor fusion with LiDAR or radar, which are lighting-independent.

- Occlusion: When one object blocks another, correspondence or tracking is temporarily lost. Mitigation: multi-frame tracking with prediction (e.g., Kalman filter) to estimate position through brief occlusions.

- Fast motion: Large inter-frame displacement can violate the small-motion assumption behind optical flow and cause motion blur. Mitigation: pyramidal (coarse-to-fine) Lucas–Kanade optical flow and higher frame-rate capture.

- Textureless regions: Flat, uniform surfaces (e.g., a plain wall or road) provide few distinctive features for matching, causing ambiguous correspondences. Mitigation: semi-global matching (SGM) for stereo, and fusing camera data with other sensors that do not rely on texture, such as LiDAR.